# EMM messages

#    00 don't show any EMMs (default)

#    01 show only valid EMMs

#    02 show valid and bad EMMs with complete data display

M: { 01 }

# ECM messages

#    00 don't show anything about ecm

#    01 normal mode, show pids processed, decoded ecm and cw

#    02 verbose mode, show valid and bad ECMs with data

C: { 01 }

# AU

#    00 disable, no emm pids processed

#    01 enable, emm pids always processed for soft-au and shared cards

#    02 auto, emm started only if channel cant be decoded

#    03 process emm pids for network cards update only

A: { 02 }

# key update (sum 01 or 02 with 04)

#    01 update only new keys, default setting

#    02 update all keys (used for valid PMK checking)

#    04 enable TPS au

U: { 05 }

# config files folder (softcam, autoroll, ignore/priority)

#    00 files in /var/keys

#    01 files in /tmp

T: { 00 }

# network mode, use summ for several clients

#    00 no network (default)

#    01 newcamd netclient

#    02 radegast netclient

#    04 camd3 netclient

#    08 gbox netclient

G: { 09 }

# network retry, use summ for several options

#    00 disable

#    01 retry every new ecm

#    02 try to connect to offline shares every Q: seconds

#    04 try to detect and fast reconnect to lost (and not used atm) server

#    + XX messages number and YY seconds to reconnect

#    mg will reconnect to server, if no answer to last XX ecm/keepalive or

#    if no answer for last ecm or keepalive sent for YY seconds

#    set to 0 to disable

N: { 07 } 5 30

# network ecm timeout in seconds

K: { 05 }

# newcamd dead routes connect retry, sec

Q: { 600 }

# network shares priority

# 00 gbox, newcamd, radegast, camd3 (default)

# 01 camd3, radegast, newcamd, gbox

# 02 newcamd, camd3, gbox, radegast

P: { 00 }

# on screen display type

#    00 No OSD (default)

#    01 neutrino

#    02 enigma

#    03 relook

#    + user password for http auth

O: { 02 } root dreambox

# on screen display Options, summ of:

#    01 show emu ecm

#    02 show network shares messages

#    04 show decoding failed / fta

#    08 show emm keys update

#    + web port to use for osd

S: { 11 } 80

# Log option, summ of:

#    00 off

#    01 network udp log

#    02 log to console

#    04 file, appended ! delete it by yourself, before it eat all your hdd

#    + IP udp-port log-file-name

L: { 03 } 192.168.0.202 8024 /tmp/mgcamd.log

# keep ecm cache, seconds

# every cache entry takes 28 bytes, so 24h cache will take *only* 240-400kb of memory,

# for openbox/elanvision users with remote shares make sense to set it higher than default value,

# to not hammer cards while timeshifting or playing crypted recordings.

E: { 15 }

# cache option, summ of:

#    00 Off (default)

#    01 Ecm pids cache, store pids used to decode in /tmp/ca_cache.list at exit, load at startup, same syntax as restore.list

#    02 Ecm data cache, remember CW for (E:) time

#    04 Emm cache for network cards, do not resend the same emm twice, cache not cleared until restart

H: { 07 }

# reread files, summ of:

#    00 No (default)

#    01 reread config file on channel change (including priority and ignore, but not replace/cache)

#    02 reread SoftCam.Key on channel change

#    04 reread SoftCam.Key if file changed

R: { 05 }

# debug, summ of

# 00 off (default)

# 01 debug ecm

# 02 debug emm

# 04 debug network ecm

# 08 debug network emm

# 16 debug network login

# 32 show mem/cpu stats every 1 min

# 64 add timestamp to log messages

D: { 127 }

# box type

# 00 autodetect, change only if you think wrong type detected.

# 01 dbox2

# 02 dreambox

# 03 triple-dragon

# 04 relook

# 05 openbox

B: { 00 }